What is this:

Change log for PYPE3_AP, Nov2024 version. This records all steps I took to install PYPE on a mordern Linux machine.

What I did:

The software is installed in Conda environment named “pype”. Python version = 3.10. Check ../ Python_Prep for details about the environment.

I used a slightly new version of ETH32 API 2.04 rather than 2.01. The software comes with an installation shell script but it won’t work on a modern Linux machine. Follow the instruction on the website and install it manually. It’s just copying some .h files to certain folders.

I found Winfort ETH32 is an old library and it only supports 32-bit archetecture. If you have a modern 64-bit machine like most people do, follow these steps for installation:

- Install DHCP server for automatic IP address configuration. (You probably already have it or it’s not necessary if you use static IP)

- You need i386 archetechture enbaled: sudo dpkg --add-architecture i386.

- You need the following 32-bit packages to let the software run:

  - sudo apt-get install libc6:i386.

  - sudo apt-get install libpthread-stubs0-dev:i386.

  - Setup your ETH32 device follow the manual instruction. Run the setting program. Find the device using a broadcast ip: such as X.X.X.255. (192.168.1.255).

Install libecV24 manually.

Edit environment as:

- PYPEDIR = ‘/usr/local/pype-python3/’

- $PYPEDIR/lib added to $PATH

- $PYPEDIR/lib, $PYPEDIR/bin, and $PYPEDIR/../p2m/ added to $PYTHONPATH.

The zip was changed to ‘.tar.xz’ to support modern Ubuntu. ./build was changed accordingly to support .tar.xz, rather than .tar.gz.

‘./build_install_pype_python3.sh’ was edited. I used this script as an installation guide and did not try to let it run automatically.

- Somba environment part was skipped. Even though it’s still in the .sh file.

- The packages were installed manually.

Edit build.sh files in src:

- All variable has been changed significantly so that they use the python under current environment, rather than the hard-coded /usr/lcoal/python one.

- Change in source: Included ‘#include <unistd.h>’ in psems.c so that it can find getpid(). Don’t understand why it was not there.

- I cleaned all warnings during the compilation. There are multiple debugging in dacq files. Such as adding a ‘retrun’ when the function psems_info() is called successfully.

Add Config file and rig parameter file:

- They need to be named properly as the machine name to be found correctly. Suppose the PC’s name is eclat, put “Config-eclat” under .pyperc/. Put “rig-eclat.par” under ./pyperc/_none.

At this point, the pype software should be able to work after the installation using Nov2024 package on a modern machine.

Chenghang 11/2024