Summary:

This document explains how PYPE works. This will be useful fur further developers.

Why you need this document:

PYPE2 was an old software. It’s not easy to understand how the software work immediately.

When trying to modifying the software so that it can be compatible with the Openiris eye tracker system, I found even though PYPE has a comprehensive API document, there is no document explaining how the software works logically. I wrote this file help myself remember what I have read and written.

Hopefully this file can help people in the future when they unfortunately have to modify the software. Especially for neuroscientists considering that some of the code are hard to understand but surprisingly functional.

Version:

PYPE3 used by Pasupathy lab is version Nov2024. I modifed the source a little so the compiling process can go smoothly.

Feb2025 version is for the awaking image system using Openiris eye tracker.

PYPE

PYPE contains several parts:

- Main part 1: UI. Tkinter is used to show all buttons, parameter tables, etc.

- Main part 2: Framebuffer. Pygame is used for visual stimulus.

- Main part 3: PYPE backend: Offers utility functions such as doing beeps, giving the reward, dealing user-input parameters, etc.

- Main part 4: DACQ. Connect PYPE with input/output devices such as ETH32, eye tracker, NI boards.

- Task files.

  - Include PYPE as ‘app’. So that you have access to all PYPE methods.

  - The structure of a task file is determined by PYPE backend. When you clicking “start” in the UI it calls a certain function in your task file.

  - Use pygame to encode stimulus and update them in the framebuffer.

  - Use fucntions in DACQ to comminicate with input/output devices.

PYPE backend:

- It contains the init() function. This is called when you start the software. Besides init the UI and framebuffer, it calls dacq_start() to init the dacq module.

- PYPE wrap some dacq functions so that PYPE and task files can use them easily.

  - This is mostly achieved by a shared memory segment. For example, if you want to fetch current eye position, PYPE fucntion will directly fetch the position that is currently saved in dacqinfo.h (defined dacq_data). Dacq module will init a chile process with “eye_tracker_server”, which continuously update dacq_data with current eye position.

  - Or it can simply call a dacq fucntion to send some digial output, such as the juice reward signal.

  - This design let your task file become independent of dacq modules. You use reward() for reward rather than dacq_juick().

DACQ:

- It defines a dacq_data shared memory segment so that everybody has access to some commonly-used data, like the eye posision, calibration parameters, etc.

- In dacq_start(), you need to let it init the servers. Usually it includes your eye tracker and an input/output device.

  - An eye tracker, such as Openeiris server, Eyelink server, or ISCAN server, will be init during the call of dacq_start(), and they continuously update eye data in dacq_data.

  - Input/output device can also have a separate server. For example, you want a digial line to receive a trigger signal so that you can init some other tasks. You can have ‘signal_trigger = 0’ in your dacq_data and having a server to keep reading the digial line and write to dacq_data -> signal_trigger.

  - For output, usually you only need to send them out once when asked. For example, juice reward signal can be sent out through ETH32 device. You can use comedi_server.c to define some fucntion such as dig_out(), and call it in dacq_juice() to send out reward signal.

- Current Feb2025 build has a confusing DACQ structure. For more detials go read the change log.